1. Product Identification and Company Information

Product Name:	High Viscosity Pink Cleaning Solution (PN 01-0013-001)

Chemical family/Product type:	Ink Jet Solution; water based; non-flammable

Manufacturer:			Spectracorp

Address:				118 South Pecos Street

Denver, Colorado, 80223

Emergency Phone Number:	303-371-6979

Fax Number:				303-371-6688

CAS Number:			Mixture/ Trade Secret

Last update:				2-21-2018

2. Hazard Identification

Pink liquid with minimal odor.  Product has little hazard if spilled and no known hazard created due to fire.

Skin:							May cause slight irritation.

Eyes:							May cause allergic reaction and irritation.

Respiratory System:				May cause irritation.

Target Organs:					Not Established.

Carcinogenicity:					No known cancer hazard.

3. Composition and Information on Ingredients

| Identification | CAS Number | Weight % |  |
| --- | --- | --- | --- |
| Water Polyethylene Glycol 600 Propylene Glycol Glycerin Isopropyl Alcohol Surfactant 485W Dispersant CT-171 Acid Red 52 Proxel GXL | 7732-18-5 25322-68-3 57-55-6 56-81-5 67-63-0 9014-85-1 68412-54-4 3520-42-1 2634-33-5 | < 70 < 30 < 10 < 10 < 10 < 3 < 3 < 0.5 < 0.5 |  |

4. First Aid

Skin:				Wash with soap and water and rinse thoroughly.

Eyes:                                   Flush eyes and under eyelids for 15 minutes with water.

Seek medical attention if irritation or redness persists.

Inhalation:	Move to fresh air. Seek medical attention if irritation persists.

Ingestion:			Give water to dilute and get medical attention.

5. Fire Fighting Measures

NFPA Rating: 	Health – 1, Flammability – 1, Reactivity – 0, Special Hazards - 0

Extinguishing Media:	Water Spray, carbon dioxide, dry chemical.

Hazardous Combustion Products:   	Material may burn in fire conditions releasing Products which may be toxic (SO2 CO,CO2, and volatile organics).

Fire Fighting Procedures:                   Wear approved MSHA/NIOSH breathing apparatus and full protective clothing.

6. Accidental Release

Spill Response:	Collect spilled material and wash area with water.

Neutralizing Chemicals:		      None Required

Waste Disposal:			      Follow all federal, state and local Regulations

7. Handling and Storage

Handling: Avoid any contact with skin, eyes and clothing, do not breath vaporized fluid, wash hands and any areas that come in contact with fluid.

Storage: Do not store in extreme heat (100 degrees or hotter) or extreme cold (40 degrees or colder).  Do not store in direct sunlight. Keep out of reach of children.

8.Exposure Controls/Personal Protection

In accordance with good industrial practices, handle with care and avoid personal contact.

Wear protective gloves, safety goggles, and other protective clothing.

Avoid contact with skin, eyes, and clothing.

Mechanical exhaust required.

Irritant.

Keep tightly closed.

Keep from freezing.

| Identification | CAS Number | OSHA NIOSH ACGIH |  |
| --- | --- | --- | --- |
| Water Polyethylene Glycol 600 Propylene Glycol Glycerin Isopropyl Alcohol Surfactant 485W Dispersant CT-171 Acid Red 52 Proprietary Antimicrobial | 7732-18-5 25322-68-3 57-55-6 56-81-5 67-63-0 9014-85-1 68412-54-4 3520-42-1 2634-33-5 | None None None None None None None None None 15 mg/m3 TWA 40 mg/m3 TWA 10 mg/m3 TWA 980 mg/m3 TWA 980 mg/m3 TWA 980 mg/m3 TWA Not Established Not Established Not Established Not Established Not Established Not Established Not Established Not Established Not Established Not Established Not Established Not Established |  |

9. Physical Data

Appearance						Pink Colored Liquid

Solubility in Water					Miscible

Freezing Point					Less than 0 degrees Celsius

Boiling Point						Greater than 100 degrees Celsius

PH (100%)						7.0 – 10.0

Specific Gravity, 25 C				1.01 - 1.10

Viscosity, cps @ 25 C	                                 3.0 - 8.0

Flash Point, C					Not Established

Volatile Organic Compounds (VOC)		None

10.Reactive Data

Reactivity:			No dangerous reaction under normal conditions known to occur.

Chemical Stability:		Stable under normal conditions.  Will not polymerize.

Incompatibility:		Strong Oxidizing agents, Strong Acids

Hazardous Decomposition Products:	Carbon monoxide, Carbon Dioxide, Sulfur Oxides.

11. Toxicological Information

The components of this product are not listed by the NTP, IARC, or regulated as a carcinogen of OSHA.  The product has not been tested for ingestion but will likely cause irritation if ingested based upon the ingredients contained in the formula.  Individual compounds are listed below in pure form.  This is a mixture and concentration levels are dramatically reduced for the fluid over the pure chemical toxicological values.

| Identification | CAS Number | Acute Toxicity Ingestion LD50 Rat Inhalation |  |
| --- | --- | --- | --- |
| Water Polyethylene Glycol 600 Propylene Glycol Glycerin Isopropyl Alcohol Surfactant 485W Dispersant CT-171 Acid Red 52 Proxel GXL | 7732-18-5 25322-68-3 57-55-6 56-81-5 67-63-0 9014-85-1 68412-54-4 3520-42-1 2634-33-5 | No information available No information available >2,000 mg/Kg Not listed 20 g/Kg Not listed 12,600 mg/kg 570 mg/m3/1 Hour 5840 mg/Kg 72.6 mg/m3/4 hours >6,300 mg/Kg No information available >2,000 mg/Kg No information available >10,300 mg/Kg No information available No information available No information available |  |

12. Ecological Information

Environmental Hazard.  There are no known environmental hazards contained in the product.  The dye in the solution can color waterways and should not be disposed of except in small quantities in sanitary sewer systems.

13. Disposal Considerations

Waste Disposal: Dispose of waste in accordance to all local, state, provincial, and national requirements.

14. Transport Information

DOT:	Proper Shipping Name: Ink Jet Ink

Non-Hazardous for Transport: This substance is considered to be nonhazardous

For transport and not regulated under DOT, IATA, ADR, or RID.

15. Regulatory Information

CALIFORNIA PROPOSITION 65: This product contains no chemicals know to the state California to cause cancer, birth defects, or other reproductive harm.

SARA Title III 311/312: This product has been reviewed according to the EPA Hazard Categories” promulgated under section 311 and 312 of SARA TITLE III, and is considered, under applicable definition, to be an immediate/acute health hazard.

EEC SYMBOLS AND INDICATIONS OF DANGER: None

R-PHRASES: R36/38 – Irritating to eyes and skin.

S-PHRASES: S24/25 – Avoid contact with skin and eyes.

WHMIS HAZARD SYMBOLS: None

TOXIC Substance Control Act Status

All ingredients are listed on the Environmental Protection Agency’s Toxic

Substance Control Act (TSCA ) Inventory.

16. Other

The above information is believed to be correct but does not purport to be all-inclusive and shall only be used as a guide. Spectracorp offers this information as a service to our customers and shall not be held liable for any damage resulting from handling or from contact with the above product.

END OF SDS